# LogiEdge: Intelligent Edge AI Platform for Cold-Chain Logistics

AIML ZG535 Machine Learning on Edge - Mini-Project - Group 18

RAVI SHANKAR NAYAK — <BITS ID: 2025AE05832>
KAPADIA VIHAR KETANKUMAR — <BITS ID: 2025AE05833>

PRASHANT V PANCHAL — <BITS ID: 2025AE05830>

SHEFALI DUTTA— <BITS ID: 2025AE05814>


GitHub:      <private repo URL — instructor added as collaborator>
Demo video:  <link>
Date:        <submission date>

# Section 1 — FreightBridge Deployment Context

### 1.1 The operational problem

FreightBridge currently relies on two-hourly handwritten temperature logs, has no predictive-maintenance capability, and uses cloud GPS that becomes unavailable on rural routes. The consequences included a ₹28 lakh vaccine loss on the Nashik-Aurangabad route, rejection of a shipment because its chain-of-custody record was incomplete, and two preventable roadside breakdowns. In every case, the common failure was that nothing on the truck was continuously watching and acting.

### 1.2 Latency

Table 1 — Cloud round-trip latency, rural 4G

| Leg | Best (s) | Worst (s) |
| --- | --- | --- |
| Sensor → edge node (I²C/SPI) | 0.005 | 0.020 |
| Edge node → cell tower (rural 4G RAN) | 0.060 | 0.250 |
| Tower → cloud region (Mumbai backhaul) | 0.030 | 0.080 |
| Cloud inference + queueing | 0.050 | 0.200 |
| Cloud → ops centre → truck ack | 0.090 | 0.330 |
| Total round trip | 0.235 | 0.880 |

The calculated rural-4G cloud round trip is 0.235 s (Table 1) in the best case and 0.880 s in the worst connected case. Therefore, cloud inference is not inherently too slow for the 90 s alert SLA. The decisive weakness is availability. In a dead zone, latency is not merely higher than 0.88 s; the round trip is undefined because no request or alert can complete. With cargo temperature capable of rising at 1 °C/min, a cloud-only design can miss the entire detection and intervention window. LogiEdge instead performs sensing, preprocessing, inference, logging, and driver alerting locally, then synchronises records when coverage returns (Figure 1).

### 1.3 Bandwidth

Table 2 — Raw telemetry vs edge-processed uplink, per truck per day

| Stream | Samples/day | MB/day |
| --- | --- | --- |
| Temperature, 1 Hz | 86,400 | 0.346 |
| Vibration, 500 Hz × 3 axes | 129,600,000 | 518.400 |
| Door events | 20 | 0.001 |
| Raw total |  | 518.747 |
| Edge-processed (alerts + heartbeat + summary) |  | 0.134 |

Reduction 3,885×. At ₹0.10/MB: ₹51.87 vs ₹0.01 per truck per day; ₹50.2 lakh/year saved across 265 trucks.

Vibration dominates the raw-data calculation (Table 2): 518.4 MB of the 518.747 MB daily total, or about 99.93%. The bandwidth case is therefore principally a vibration-processing case rather than a temperature case. The 500 Hz x 3-axis rate in Task A1 describes raw accelerometer acquisition for the transmission estimate, whereas the 0.5 Hz vibration RMS stream in Task C1 is a derived signal after local sensor-side aggregation. Computing RMS before publication already reduces 1,500 raw axis samples per second to roughly one value every two seconds, a 3,000x reduction. Edge feature extraction and event-only uplink then reduce the remaining traffic to 0.134 MB/day, avoiding about ₹50.2 lakh annually across 265 trucks.

### 1.4 Connectivity

Seven documented route locations lose cellular coverage for 35-90 minutes, creating up to 10.5 hours of cloud invisibility in the stated worst case. At a 1 °C/min failure rate, refrigerated cargo can leave the 2-8 °C envelope about four minutes into a gap. Its temperature then approaches the 35-42 °C cabin ambient, reaching near-equilibrium after roughly 31 minutes rather than increasing without limit. The architectural requirement is therefore temporal: detection and local escalation must occur before connectivity resumes, while logs are queued for later synchronisation.

### 1.5 Privacy

LogiEdge applies data minimisation: raw sensor telemetry remains on the truck, while classifications, alerts, health summaries, and custody evidence are synchronised. This reduces the exposed data surface, removes a third-party cloud inference processor from the real-time path, supports tenant isolation, and permits an auditable local custody log under the DPDP Act context. However, local inference alone cannot guarantee that data 'cannot be accessed.' That warranty also requires encryption at rest and in transit, device identity, authenticated services, least-privilege access, secure boot, key protection, signed updates, retention controls, and auditable administrative access.

### 1.6 Hardware selection

Table 3 — Constraint Triangle inputs

| Option | ₹/truck | TDP | Pilot (85) | Fleet (265) | Fits 10 W? |
| --- | --- | --- | --- | --- | --- |
| Pi 5 8 GB + AI HAT+ (13 TOPS) | 15,000 | 7.5 W | ₹12.75 L | ₹39.75 L | Yes |
| Jetson Orin Nano Super (67 TOPS) | 45,000 | 15 W | ₹38.25 L | ₹119.25 L | No |
| STM32H7 custom MCU | 3,500 | 0.4 W | ₹2.98 L | ₹9.28 L | Yes |

Dominant vertex: cost, with power as a hard feasibility gate.

The recommended platform is the Raspberry Pi 5 with AI HAT+ (Table 3). It satisfies the 10 W power gate at 7.5 W, costs ₹12.75 lakh for the 85-truck pilot and ₹39.75 lakh for 265 trucks, and provides enough Linux, container, MQTT, monitoring, and accelerator capability for the full platform. The Jetson Orin Nano is rejected first on feasibility because its 15 W moderate-load TDP exceeds the budget, before considering its threefold unit cost. The STM32H7 could execute the 803-parameter INT8 classifier, but it cannot readily host the required Docker, Mosquitto, Python/TFLite pipeline, Ansible-managed deployment, PSI monitoring, and flexible OTA stack. Building those capabilities as custom firmware would increase engineering, integration, diagnostic, and lifecycle cost across the fleet.

### 1.7 Roofline

AI       = 45 × 10⁶ FLOP / 18 × 10⁶ B  = 2.500 FLOP/byte
I_ridge  = 16 × 10⁹ / 12 × 10⁹          = 1.333 FLOP/byte
AI > I_ridge                            → COMPUTE-BOUND
attainable = min(16, 2.5 × 12 = 30)     = 16 GFLOP/s
compute 2.812 ms vs memory 1.500 ms     (1.88×)

The supplied Roofline inputs give an arithmetic intensity of 2.5 FLOP/byte, above the 1.333 FLOP/byte ridge point, so the stated workload is compute-bound with a 16 GFLOP/s attainable ceiling. The appropriate optimisation direction is therefore to reduce arithmetic through quantisation and structured pruning, or offload supported operations to the NPU. Memory-oriented techniques such as cache blocking, tiling, or data-layout tuning raise effective bandwidth but do not attack the active roof. This result must be interpreted cautiously: the brief assumes 45 MFLOP and 18 MB, while the implemented 803-parameter MLP is about 1.6 kFLOP and measured near 1.2 µs, not the 2.8 ms implied by the supplied figures.

Figure 1 — LogiEdge system architecture

# Section 2 — Sensor Pipeline and MQTT Design

### 2.1 Why a broker on the truck

A local MQTT broker provides three architectural benefits. First, it isolates sensor acquisition, preprocessing, inference, monitoring, alerting, and synchronisation as separate processes, so a failure in a non-critical consumer does not stop the local detection path. Second, it supports several independent consumers of the same stream without point-to-point coupling. Third, the deployment boundary is already message-oriented: truck services publish locally and a synchronisation component forwards selected records when connectivity exists. Mosquitto is bound to localhost, preventing raw sensor topics from being directly exposed on the cellular interface and strengthening the data-minimisation design.

### 2.2 Topic tree and QoS

Figure 2 — MQTT topic tree

logibridge/trucks/{truck_id}/
    ├── sensors/temperature      QoS 0   1 Hz
    ├── sensors/vibration        QoS 0   0.5 Hz
    ├── events/door              QoS 1   discrete, retained
    ├── inference                QoS 1   0.1 Hz
    ├── alerts                   QoS 2   on Warning or Critical
    └── status/{drift,health}    QoS 1 / QoS 0 retained

Table 4 — QoS justification

| Topic | QoS | Why |
| --- | --- | --- |
| sensors/temperature | 0 | Superseded 1 s later; the 5-sample MA and 30-sample window make any single reading near-irrelevant |
| sensors/vibration | 0 | Same at 0.5 Hz; losing one of 15 window samples moves RMS less than sensor noise |
| events/door | 1 | Discrete, not superseded; retained so a restarting consumer learns current state |
| inference | 1 | PSI’s rolling window must not silently lose entries; duplicates deduplicated on seq |
| alerts | 2 | The only topic worth a four-step handshake — a duplicate could dispatch a second escalation |
| status/health | 0, retained | Heartbeat superseded every 5 min; retained so the dashboard shows last-known state during a dead zone |

The QoS pattern follows consequence rather than importance in the abstract (Table 4): QoS rises as the cost of losing one message increases, and falls as publication rate and natural replacement increase. High-rate temperature and vibration readings use QoS 0 because the next sample arrives quickly and windowing reduces the influence of one loss. Door events and inference results use QoS 1 because they are not safely superseded; sequence numbers handle duplicates. Alerts use QoS 2 because neither loss nor duplicate escalation is acceptable. Placing truck_id above the data category supports both required subscription views (Figure 2): a truck-local service can subscribe to logibridge/trucks/{truck_id}/# for all local streams, while the operations backend can subscribe to logibridge/trucks/+/alerts and related status topics across the fleet.

### 2.3 Preprocessing

5-sample causal moving average → 30 s window, 10 s step → 6 features (temp_mean, temp_std, temp_roc °C/min, vib_rms, vib_peak, vib_kurtosis) → z-score against frozen statistics.

Table 5 — Frozen normalisation statistics (61 clean windows, 10 min)

| Feature | Mean | Std |
| --- | --- | --- |
| temp_mean | 4.0950 | 0.2063 |
| temp_std | 0.1410 | 0.0457 |
| temp_roc | 0.0137 | 0.5118 |
| vib_rms | 0.4513 | 0.0122 |
| vib_peak | 0.4915 | 0.0210 |
| vib_kurtosis | −0.3430 | 1.2618 |

The five-sample moving average is causal because the same implementation must run online, where future samples are unavailable. A centred filter would create look-ahead leakage during training and could not be reproduced during deployment. Temperature rate-of-change is estimated with a least-squares slope over the window rather than an endpoint difference, making it less sensitive to noise in two individual samples and using all observations. Normalisation statistics are computed from clean training data and frozen in training_stats.npy. Recomputing mean and standard deviation from live data would let a gradual temperature failure move the baseline with the cargo, progressively normalising away the deviation. The measured check is consistent with theory: raw σ=0.3 becomes 0.141 after averaging (Table 5), close to 0.3/sqrt(5)=0.134.

### 2.4 Normalisation experiment (mandated 3σ test)

Table 6 — Sensitivity to corrupted statistics (M2, 57 validation windows)

| Condition | Accuracy | Δ | Recall N | Recall W | Recall C |
| --- | --- | --- | --- | --- | --- |
| correct | 96.49% | — | 91.3% | 100.0% | 100.0% |
| mean +3σ | 91.23% | −5.26 | 100.0% | 70.6% | 100.0% |
| mean −3σ | 94.74% | −1.75 | 87.0% | 100.0% | 100.0% |
| std ×3 | 94.74% | −1.75 | 95.7% | 88.2% | 100.0% |

The 3σ experiment exposes a class-specific failure that headline accuracy hides (Table 6). Critical recall remains 100% under all corruptions because Critical windows contain a large vibration signature and are well separated. Normal and Warning are much closer, approximately one temperature standard deviation apart, so shifting the mean by +3σ moves Warning samples toward the learned Normal region. Warning recall consequently falls to 70.6% while overall accuracy still appears acceptable at 91.23%. This is silent normalisation blindness: the service continues to publish plausible, confident outputs, but becomes selectively insensitive to an early-stage condition. It is harder to detect than a crash because health checks remain green and Critical cases may still pass. training_stats.npy must therefore be versioned with the model, integrity-checked, content-hashed, deployed atomically, and monitored as a safety-relevant model artefact rather than treated as replaceable configuration.

### 2.5 Data fusion

LogiEdge uses feature-level fusion. Temperature and vibration are filtered and summarised independently, then aligned over the same 30 s window and concatenated into one six-value vector for a single classifier. Data-level fusion would require resampling two streams arriving at 1 Hz and 0.5 Hz onto a common timeline, handling missing points, and carrying many more values into the model without adding useful temporal resolution. Decision-level fusion would train separate temperature and vibration classifiers and combine their labels, but this loses the joint interaction that distinguishes Warning from Critical. In particular, Class 1 may be a temperature drift or early anomaly, whereas Class 2 may be identified by the combined magnitude and pattern. A per-sensor classifier cannot directly learn that cross-sensor boundary.

### 2.6 Bounded drift — a documented design decision

The mandated +0.08 °C per reading at 1 Hz equals +4.8 °C/min. If applied without bound for the full 15-minute Warning capture, it would add 72 °C, causing almost every window after the first minute to satisfy the problem statement's Critical threshold rather than Warning. The simulator therefore preserves the specified ramp exactly but introduces a recovery cycle: temperature rises at +0.08 °C per reading to a bounded deviation and then cools before the next cycle. This maintains repeated early-drift examples without relabelling extreme breaches as Warning. In the resulting dataset, 64.2% of Warning samples lie in the intended 1-3 °C band and only 5.8% exceed 3 °C.

# Section 3 — Model Deployment and Pipeline Mapping

### 3.1 Dataset and the accuracy gate

291 windows — 117 Normal / 87 Warning / 87 Critical. MLP 6→32→16→3, 803 parameters.

Table 7 — M1 validation (random split, n=57)

| True Predicted | Normal | Warning | Critical | Recall |
| --- | --- | --- | --- | --- |
| Normal | 21 | 2 | 0 | 91.3% |
| Warning | 1 | 16 | 0 | 94.1% |
| Critical | 0 | 0 | 17 | 100.0% |

94.74% against an 88% gate. Blocked split (no window overlap): 100.00%.

The resulting dataset contains 291 windows: 117 Normal, 87 Warning, and 87 Critical. The 6-32-16-3 MLP achieved 94.74% on the 57-window random validation split, exceeding the 88% gate (Table 7). It is not artificially perfect because the three errors occur at a genuine class boundary: all are at 4.3-4.6 °C with slowly rising temperature. Two door-opening windows resemble Warning, while one early fault-onset window still resembles Normal using only the six mandated features. Most importantly, no Critical window is missed. Both random and blocked results are reported: the random split measures the submitted training setup, while the non-overlapping blocked split checks whether overlap between adjacent sliding windows is inflating performance.

### 3.2 The 10-stage Edge ML pipeline

LogiEdge follows the ten-stage Edge ML lifecycle end to end (Table 8). simulator.py and generate_dataset.py collect labelled Normal, Warning, and Critical streams. preprocessing.py applies causal filtering, 30 s windowing, six-feature extraction, and frozen normalisation. train_model.py selects and trains the 6-32-16-3 MLP, while the validation logic records confusion matrices and the 88% gate. convert_ptq.py creates the M2 full-INT8 model, and prune_quantise.py produces M3 using structured pruning followed by PTQ. benchmark.py compares latency, p95, size, accuracy, energy estimate, and Critical recall. The TFLite model, training_stats.npy, and reference distribution are packaged with inference_service.py in the Docker image. Deployment uses the Dockerfile and logibridge_deploy.yml. At runtime, inference_service.py publishes classifications and alerts, while drift_monitor.py evaluates rolling PSI against reference_dist.json and reports drift and recovery.

Table 8 — LogiEdge mapped to the 10-stage Edge ML pipeline

| # | Stage | LogiEdge implementation |
| --- | --- | --- |
| 1 | Data collection | simulator.py publishes temperature, vibration RMS, and door events; generate_dataset.py captures labelled windows for the three operating classes. |
| 2 | Preprocessing | preprocessing.py applies five-sample causal moving averages, 30-second windows with 10-second steps, and frozen z-score normalisation from training_stats.npy. |
| 3 | Feature engineering | preprocessing.py computes temperature mean, standard deviation and slope plus vibration RMS, peak and kurtosis, then concatenates the six values. |
| 4 | Model selection | train_model.py defines the compact 6-32-16-3 ReLU MLP and compares it against the 88% validation gate. |
| 5 | Training | train_model.py trains on the generated Normal, Warning, and Critical windows and saves the baseline model and preprocessing artefacts. |
| 6 | Evaluation | Validation code produces accuracy, confusion matrices, per-class recall, a random split, and a non-overlapping blocked split. |
| 7 | Optimisation | convert_ptq.py creates M2 full INT8; prune_quantise.py applies structured pruning with PolynomialDecay and then INT8 PTQ for M3. |
| 8 | Conversion / packaging | TFLite models, training_stats.npy, inference_service.py, and requirements are packaged by the Dockerfile with the model in the final cacheable layer. |
| 9 | Deployment | logibridge_deploy.yml installs artefacts and converges the inference container; the canary strategy deploys first to 10 trucks. |
| 10 | Monitoring | drift_monitor.py compares rolling P(Normal) PSI with reference_dist.json, while MQTT health, alerts, and local logs support operations. |

### 3.3 Docker layer cache

Table 9 — Measured layer sizes (docker history)

| Layer | Size |
| --- | --- |
| RUN pip install -r requirements.txt | 89.6 MB |
| COPY preprocessing.py | 24.6 kB |
| COPY simulator.py | 24.6 kB |
| COPY inference_service.py | 20.5 kB |
| COPY training_stats.npy | 12.3 kB |
| COPY model.tflite | 12.3 kB |

Step 2 of the demonstration: layers 2–8 CACHED, [9/9] COPY model.tflite rebuilt in 0.2 s. pip install took 10.0 s in Step 1 and zero in Step 2.

Table 10 — OTA bandwidth, 85-truck pilot

|  | Per truck | Fleet (85) | Cost @ ₹0.10/MB |
| --- | --- | --- | --- |
| Full image | 188.000 MB | 15,980 MB | ₹1,598.00 |
| Model layer only | 0.003936 MB | 0.335 MB | ₹0.03 |
| Saved |  | 15,979.7 MB | ₹1,597.97 |

Reduction factor 47,764×.

The Dockerfile builds on python:3.11-slim and places the stable, expensive dependency layer before application and model copies, with MODEL_PATH exposed as an environment variable so M1, M2 and M3 can be switched without rebuilding the image. requirements.txt is installed first, allowing the 89.6 MB pip layer to remain cached when only model.tflite changes. In the demonstrated rebuild, layers 2-8 were reused and only the final model copy rebuilt in 0.2 s (Table 9); the 10.0 s pip installation disappeared from the second build. Reversing COPY model.tflite and pip install would invalidate the dependency layer for every model release, forcing repeated package download, installation, image creation, and distribution. For 85 trucks, the measured uncompressed comparison is 15,980 MB for the full image versus 0.335 MB for the model layer (Table 10). Real registries transfer compressed layers, so absolute traffic will differ, but the layer-isolation ratio remains the relevant design result.

### 3.4 OTA strategy

Table 11 — Per update cycle (280 KB model, 85 trucks, ₹0.10/MB). Shadow mode carries the same 23.80 MB model download plus 0.7221 MB per truck of parallel-inference comparison telemetry returned over the cycle: 23.80 + 85 × 0.7221 = 85.18 MB.

| Strategy | MB/cycle | ₹/cycle | ₹/year | Blast radius |
| --- | --- | --- | --- | --- |
| Full replacement | 23.80 | 2.38 | 20.65 | all 85 |
| Canary (10 first) | 23.80 | 2.38 | 20.65 | 10, then 75 |
| Shadow mode | 85.18 | 8.52 | 73.82 | 0 (advisory) |

Canary abort: 2.80 MB = ₹0.28 instead of ₹2.38.

Canary deployment is recommended (Table 11). The transfer cost of all three strategies is negligible compared with a ₹28 lakh spoilage event, so safety and recoverability decide the choice. Full replacement exposes all 85 trucks to the same defect before field evidence is available. Rollback is difficult on these routes because an affected truck may enter a 35-90 minute dead zone after receiving the update, preventing immediate remote correction while cargo remains in transit. A ten-truck canary limits the initial blast radius and costs only ₹0.28 if aborted before the remaining 75 trucks. Shadow mode is technically safest because it makes the new model advisory, but it requires parallel inference, comparison telemetry, additional operational logic, and 85.18 MB per cycle. For this small, fast model and constrained pilot, that complexity delays actionable improvement without a proportionate risk reduction beyond a controlled canary.

# Section 4 — Optimisation and Pareto Analysis

### 4.1 Benchmark results

Table 12 — Five-metric benchmark (200 timed runs × 5 blocks after 10 warm-up)

| Variant | Mean (ms) | p95 (ms) | Size (KB) | Accuracy (%) | Energy (mJ) | Class 2 recall |
| --- | --- | --- | --- | --- | --- | --- |
| M1 FP32 baseline | 0.0016 | 0.0014 | 5.26 | 94.74 | 0.02206 | 100% |
| M2 PTQ INT8 | 0.0015 | 0.0014 | 4.55 | 96.49 | 0.02224 | 100% |
| M3 Pruned 35% + INT8 | 0.0012 | 0.0012 | 3.84 | 98.25 | 0.02191 | 100% |

Figure 3 — Pareto frontier across latency, size and accuracy (M1, M2, M3)

M3, the 35% structured pruned plus INT8 model, is the only non-dominated measured option (Table 12, Figure 3). It has the lowest mean and p95 latency, smallest file, highest validation accuracy, lowest estimated energy, and the same 100% Critical recall. Consequently, there is no measured trade-off curve to navigate: M1 and M2 are worse on every reported axis, and the Pareto frontier collapses to the single M3 point. This does not mean every pruned INT8 model will dominate its baseline; it means that, for this trained instance and benchmark, no metric must be sacrificed to select M3. The deployment decision can therefore be based on robustness and footprint rather than balancing latency against accuracy.

The benchmark differences require restraint. Mean latencies differ by only fractions of a microsecond and are close to timer, scheduling, and runtime noise. Measurements were taken on an x86 host with AVX-512 rather than the Raspberry Pi 5 target, whose NEON path has different kernels: INT8 can use SDOT instructions, while FP32 throughput is materially lower. The reported 98.25% accuracy is 56 correct windows out of 57, only one window better than M2, and is not evidence of a statistically meaningful generalisation gain. Before deployment, the complete benchmark should be repeated on the selected Pi 5 and AI HAT+ environment, using longer runs, pinned execution conditions, target TFLite delegates, repeated dataset splits, thermal soak, and direct power measurement. The target result, not the development laptop result, should control release approval.

### 4.2 Pruning structure

Table 13 — M3 structure after pruning

|  | Before | After | Removed |
| --- | --- | --- | --- |
| dense_0 units | 32 | 21 | 34.4% |
| dense_1 units | 16 | 10 | 37.5% |
| Total units | 48 | 31 | 35.4% |
| Parameters | 803 | 400 | 50.2% |

Structured pruning removes complete neurons or channels and then rebuilds smaller dense tensors. Here, hidden layers shrink from 32 and 16 units to 21 and 10, reducing parameters from 803 to 400 (Table 13). That reduction can lower actual matrix dimensions, file size, and executed operations. Unstructured pruning merely sets individual weights to zero; without a sparse kernel and sparse storage format, the CPU still loads and multiplies the original dense tensors, so zeros buy little on this hardware. PolynomialDecay does not itself choose the final compact architecture. During training it gradually increases pruning from the initial sparsity toward the 35% target, allowing the remaining connections to adapt rather than deleting capacity abruptly; the compact model is then exported and quantised.

### 4.3 Deployment recommendation

SLA decomposition

window fill (30 s window)           30,000 ms
step alignment (worst case)          10,000 ms
local log + MQTT alert publish          100 ms
----------------------------------------------
remaining for inference              49,900 ms
slowest variant uses                   0.0016 ms

Deploy M3 to the 85-truck pilot through the canary process. Latency is not the differentiator: even the slowest measured variant uses 0.0016 ms, while the 90 s SLA leaves 49,900 ms after window fill, worst-case step alignment, logging, and MQTT publication. Flash and SRAM are not binding either: the 3.84 KB model and its 400 parameters are a negligible fraction of the Raspberry Pi 5's microSD storage and 8 GB LPDDR4X, so memory fit does not discriminate between the variants. The safety gate is Critical-class recall, and all variants achieve 100%, above the required 95%. M3 is preferred because it preserves that recall with the smallest 3.84 KB file, half the baseline parameter count, and the lowest measured energy estimate. A smaller model is easier to transfer reliably during six-weekly OTA cycles, reduces update exposure over intermittent cellular links, shortens verification and rollback payloads, and leaves more storage margin for dual-version deployment and local logs.

# Section 5 — MLOps Monitoring and Reflection

### 5.1 PSI drift monitoring

Reference: 300 clean Normal windows, P(Normal), 4 bins → [0.003, 0.253, 0.587, 0.157]

Table 14 — PSI across the injection cycle (32 evaluations, 100-inference rolling window)

| Phase | PSI range | Alert |
| --- | --- | --- |
| 1 — clean baseline | 0.014 – 0.050 | none |
| 2 — --anomaly combined injected | 0.191 → 13.909 | [LOGIBRIDGE DRIFT ALERT] |
| 3 — clean restored | 11.101 → 0.015 | [LOGIBRIDGE DRIFT RECOVERED] |

At peak the distribution fully saturates: bins [1.00, 0.00, 0.00, 0.00]. Detection: two evaluation cycles (~29 s). Recovery: ~1 min 50 s.

During clean operation, P(Normal) is spread across the reference bins [0.003, 0.253, 0.587, 0.157], producing PSI values of 0.014-0.050 (Table 14). After the combined anomaly is injected, PSI crosses the 0.25 alert threshold within two evaluation cycles — 0.191, then 0.939 roughly 29 seconds later. Over a further eight evaluations, probability mass migrates out of the upper Normal-confidence bins until the distribution saturates at [1.00, 0.00, 0.00, 0.00] and PSI reaches 13.909. After clean data returns, recovery is slower because the 100-inference rolling buffer still contains anomalous scores; PSI falls from 11.101 to 0.015 only as those entries are flushed, taking about 1 min 50 s. P(Normal) crossed after 12 inferences versus 20 for maximum-softmax confidence, so P(Normal) was selected for earlier, directionally meaningful detection.

The injected event is best treated as a real physical change in the monitored process that manifests as covariate shift in sensor features and a corresponding shift in model outputs. It is not label drift, and it does not establish concept drift because the relationship between refrigeration condition and class definition has not changed. Therefore, the alert should trigger operational investigation and cargo protection, not automatic retraining. Retraining would only be justified if sustained drift remains after equipment recovery and data-quality checks, with reviewed labels showing that the deployed feature-to-class relationship has genuinely changed.

### 5.2 Ansible idempotency

RUN 1:  ok=7   changed=5   failed=0   skipped=0
RUN 2:  ok=6   changed=0   failed=0   skipped=1

The second Ansible run demonstrates idempotency because each task converges on declared state. File-copy tasks compare checksums and report changed only when model or reference content differs. Their registered results gate the container-stop task, so it is skipped when neither artefact changes. The docker_container task declares the desired image, environment, mounts, restart policy, and running state; on the second run it observes that the container already matches instead of issuing another start action. The final verification reads container status but uses changed_when: false because observation is not a configuration change. The result is changed=5 on the first application and changed=0 on the unchanged repeat.

### 5.3 One genuine technical difficulty

A genuine difficulty occurred when the PSI monitor raised a value of 4.96 on clean baseline traffic, although a value near zero was expected. Comparing the live four-bin distribution with reference_dist.json showed that the two distributions had been produced by different model contents even though both files were named model.tflite. The filename therefore gave a false appearance of consistency. The deployment and reference-generation workflow was changed to identify the model by a content hash and associate the reference distribution with that hash. At startup, the monitor now verifies the deployed model identity before accepting the reference. This prevents a valid reference from being silently paired with the wrong model variant.

### 5.4 One architectural change

The principal architectural change would be to include door state in classification. Two of the three random-split errors were door-opening windows classified as Warning, showing that temperature transients caused by normal loading activity are currently indistinguishable from refrigeration drift. The extractor already receives door events, but the mandated six-feature vector cannot represent them. A production revision would add a seventh feature such as door-open fraction or time-since-close. A simpler alternative is a reviewed gating rule that suppresses a Warning during an OPEN event and for a short, bounded recovery interval after CLOSE, while never suppressing vibration-based Critical escalation.

### 5.5 Known limitations

Known limitations are explicit. Energy is calculated from CPU utilisation and a laptop TDP estimate, not measured electrically at the edge device. Latency was measured on x86 rather than Raspberry Pi 5. With only 57 validation windows, one sample changes accuracy by 1.75 percentage points. Kurtosis computed from roughly 15 vibration samples per window is statistically noisy. Sliding windows are correlated even though a blocked split was also reported. Finally, door state is collected and tracked but not included in the six-feature classifier, creating avoidable ambiguity around loading events. These limitations require target-device validation before operational approval.